Lab Practical #11:

Study the concept of routing using packet tracer. (Static Routing)

Practical Assignment #11:

- Connect the two different routers based on the calculated IP addresses and subnet using a packet tracer.

- To connect two different routers using Cisco Packet Tracer and configure static routing to enable communication between different networks.

IP Address Table

| Device | Interface | IP Address | Subnet Mask | Network ID |
| --- | --- | --- | --- | --- |
| 192.168.1.1 | FastEthernet0 | 192.168.1.1 | 255.255.255.0 | 192.168.1.0 |
| 192.168.1.2 | FastEthernet0 | 192.168.1.2 | 255.255.255.0 | 192.168.1.0 |
| Router 0 | FastEthernet0/0 | 192.168.1.3 | 255.255.255.0 | 192.168.1.0 |
| Router 0 | Serial2/0 | 20.0.0.1 | 255.0.0.0 | 20.0.0.0 |
| Router 1 | Serial2/0 | 20.0.0.2 | 255.0.0.0 | 20.0.0.0 |
| Router 1 | FastEthernet0/0 | 10.0.0.3 | 255.0.0.0 | 10.0.0.0 |
| 10.0.0.1 | FastEthernet0 | 10.0.0.1 | 255.0.0.0 | 10.0.0.0 |
| 10.0.0.2 | FastEthernet0 | 10.0.0.2 | 255.0.0.0 | 10.0.0.0 |

Network ID Table

| Network | Network ID | Subnet Mask |
| --- | --- | --- |
| Left LAN | 192.168.1.0 | 255.255.255.0 |
| Router-to-Router Link | 20.0.0.0 | 255.0.0.0 |
| Right LAN | 10.0.0.0 | 255.0.0.0 |

Static Routing Configuration

For communication between the two different LAN networks, static routes are configured on both routers.

Router0:

- Destination Network: 10.0.0.0

- Subnet Mask: 255.0.0.0

- Next Hop: 20.0.0.2

Router1:

- Destination Network: 192.168.1.0

- Subnet Mask: 255.255.255.0

- Next Hop: 20.0.0.1

Routing Table

PDU Transfer

Result

- The two routers were successfully connected using Cisco Packet Tracer.

- The IP addresses and subnet masks were configured for all devices.

- Static routing was successfully configured between the 192.168.1.0/24 and 10.0.0.0/8 networks.

- Communication between the different networks was successfully verified using the Packet Tracer PDU.

- Connect the three different routers based on the calculated IP addresses and subnet using a packet tracer.

- To connect three different routers using Cisco Packet Tracer and configure static routing to enable communication between different networks.

IP Address Table

| Device | Interface | IP Address | Subnet Mask | Network ID |
| --- | --- | --- | --- | --- |
| 20.0.0.1 | FastEthernet0 | 20.0.0.1 | 255.0.0.0 | 20.0.0.0 |
| 20.0.0.2 | FastEthernet0 | 20.0.0.2 | 255.0.0.0 | 20.0.0.0 |
| Router 2 | FastEthernet0/0 | 20.0.0.3 | 255.0.0.0 | 20.0.0.0 |
| Router 2 | Serial2/0 | 50.0.0.1 | 255.0.0.0 | 50.0.0.0 |
| Router 3 | Serial2/0 | 50.0.0.2 | 255.0.0.0 | 50.0.0.0 |
| Router 3 | FastEthernet0/0 | 30.0.0.3 | 255.0.0.0 | 30.0.0.0 |
| 30.0.0.1 | FastEthernet0 | 30.0.0.1 | 255.0.0.0 | 30.0.0.0 |
| 30.0.0.2 | FastEthernet0 | 30.0.0.2 | 255.0.0.0 | 30.0.0.0 |
| Router 3 | Serial3/0 | 60.0.0.1 | 255.0.0.0 | 60.0.0.0 |
| Router 4 | Serial2/0 | 60.0.0.2 | 255.0.0.0 | 60.0.0.0 |
| Router 4 | FastEthernet0/0 | 40.0.0.3 | 255.0.0.0 | 40.0.0.0 |
| 40.0.0.1 | FastEthernet0 | 40.0.0.1 | 255.0.0.0 | 40.0.0.0 |
| 40.0.0.2 | FastEthernet0 | 40.0.0.2 | 255.0.0.0 | 40.0.0.0 |

Network ID Table

| Network | Network ID | Subnet Mask |
| --- | --- | --- |
| First LAN | 20.0.0.0 | 255.0.0.0 |
| Router-to-Router Link 1 | 50.0.0.0 | 255.0.0.0 |
| Second LAN | 30.0.0.0 | 255.0.0.0 |
| Router-to-Router Link 2 | 60.0.0.0 | 255.0.0.0 |
| Third LAN | 40.0.0.0 | 255.0.0.0 |

Static Routing Configuration

For communication between the three different LAN networks, static routes are configured on all three routers.

Router2:

- Destination Network: 30.0.0.0

- Subnet Mask: 255.0.0.0

- Next Hop: 50.0.0.2

- Destination Network: 40.0.0.0

- Subnet Mask: 255.0.0.0

- Next Hop: 50.0.0.2

Router3:

- Destination Network: 20.0.0.0

- Subnet Mask: 255.0.0.0

- Next Hop: 50.0.0.1

- Destination Network: 40.0.0.0

- Subnet Mask: 255.0.0.0

- Next Hop: 60.0.0.2

Router4:

- Destination Network: 20.0.0.0

- Subnet Mask: 255.0.0.0

- Next Hop: 60.0.0.1

- Destination Network: 30.0.0.0

- Subnet Mask: 255.0.0.0

- Next Hop: 60.0.0.1

Routing Table

PDU Transfer

Result

- The three routers were successfully connected using Cisco Packet Tracer.

- The IP addresses and subnet masks were configured for all devices.

- Static routing was successfully configured between the 20.0.0.0/8, 30.0.0.0/8, and 40.0.0.0/8 networks.

- Communication between the different networks was successfully verified using the Packet Tracer PDU.